# Module _ DppApi Introduction

Filename: DppApi.h

Description

Interface to the DPP Application functions.

The DP4/PX4/DP5 Digital Pulse Processor API (DppApi) is the Application Programming Interface Library for Amptek DP4, PX4 and DP5 Digital Pulse Processors (DPPs).

Copyright (c)2005, 2007, 2009 Amptek, All Rights Reserved

Developer Notes

Application programs invoke DppApi functions to establish communication with DPP, control its operation, and retrieve data. DppApi is distributed in two forms, as an object link library (.LIB) or Dynamic Link Library (.DLL).    The application executable programs (.EXE) are created by linking the application code with one of these libraries statically or dynamically (see Microsoft documentation about program linking). The applications can be developed in C, Visual Basic or many other languages.

CE Configuration files and spectra are UNICODE. XP Configuration files and spectra are ASCII. Windows NotePad can be used to convert ASCII files to/from UNICODE.

Version: v20050729_1423

### _ DppApi Introduction

DPP_BOOT_STATUS

DPP_CONFIG_SETTINGS

DPP_STATUS

# Module Configuration Helper Functions

Filename: DppApi.h

Description

help in the display, control and conversion of DPP configuration data.

All of the configuration settings have underlying support classes. Configuration Helper Functions tap some of the functionality of the underlying classes.    The goal is to reduce the amount of support coding necessary to a minimum.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

### Configuration Helper Functions

GetCoarseGainSpinFromText

GetCoarseGainTextFromSpin

GetOutputOffsetSpinFromText

GetOutputOffsetTextFromSpin

GetPresetTimeStringFromVal

GetPresetTimeValFromString

GetSlowThreshSpinFromChannel

GetSlowThreshSpinFromPercent

GetSlowThreshStringsFromSpin

GetTimeToPeakDecimationFromSpin

GetTimeToPeakSpinFromValues

# Module Configuration Transfer Functions

Filename: DppApi.h

Description

Configuration Transfer Functions provide device configuration services.

Configuration Transfer Functions sets and transfers configurations to and from the current DPP, the DppApi and selected Amptek DPP configuration files.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

Developer Notes

The DppApi and the current DPP must be configured before data acquisition operations can begin.

### Configuration Transfer Functions

Get80MHzMode

GetConfigFromBuffer

GetConfigFromDpp

GetConfigFromFile

GetConfigString

GetTempConfigSettings

GetTempConfigString

SaveConfigToFile

SendConfigToBuffer

SendConfigToDpp

SetFPGAClockDefault

SetTempConfigSettings

# Module Data Acquisition Functions

Filename: DppApi.h

Description

Control data acquisition.

Spectrum information is read as a block of data and is converted into histogram ready display values.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

### Data Acquisition Functions

ClearDppData

GetDppData

PauseDppData

ProcessDppData

# Module DP5 Text Functions

Filename: DppApi.h

Description

Sets and reads text stored in the DP5.

DP5 Text Functions works for DP5 only.

Copyright (c)2005, 2009 Amptek, All Rights Reserved

### DP5 Text Functions

GetDP5Text

SetDP5Text

# Module DPP Tune Functions

Filename: DppApi.h

Description

Tunes the fast threshold (DP4/PX4/DP5) and input offset (PX4/DP5 only).

PX4 with FPGA/Firmware greater than Version 3.13 or DP4 with FPGA/Firmware greater than Version 4.00 required.

Developer Notes

The source must be removed to auto tune the fast threshold.    Status flags are provided to indicate a tuning function successful completion. (See DPP_STATUS.)

Copyright (c)2005-2009 Amptek, All Rights Reserved

### DPP Tune Functions

TuneFastThreshold

TuneInputOffset

TuneInputOffsetInit

# Module DPP USB Additional Functions

Filename: DppApi.h

Description

Send Dpp USB Vendor Request (DP4/PX4/DP5).

Developer Notes

Additonal functions are available through Dpp USB Vendor Requests. This is an advanced function and is not supported.    See DP4/PX4/DP5 user manuals for Vendor Request function details.

Copyright (c)2005-2009 Amptek, All Rights Reserved

### DPP USB Additional Functions

SendDppVendorRequest

# Module DppApi Constructor Functions

Filename: DppApi.h

Description

Opens the DPP Application functions.

DppApi Constructor Functions create, initialize and close the DppApi.    Opening the DppApi with OpenDppApi creates and initializes an instance of the DppApi.    The DppApi instance manages and stores DPP data, configurations, and controls communication.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

Developer Notes

All other DppApi functions need the DppApi to be opened before being called.    When all DppApi operations are completed and before the application is closed, the DppApi must be closed with CloseDppApi.

### DppApi Constructor Functions

CloseDppApi

OpenDppApi

# Module DppApi Setup Notes

Filename: DppApi.h

Description

Additional setup notes.

Application programs invoke DppApi functions to establish DPP communications, control DPP operations, and retrieve spectrum data. The DppApi is distributed with an import library (.LIB), and a dynamic link library (.DLL) and a type library (.TLB).

Applications can be developed with the DppApi in C, Visual Basic and many other programming languages.

The C language representation of DppApi functions are defined in the DppApi.h header file.    These files are provided with the DppApi libraries.    The USB drivers provided with your Amptek DPP must be installed and the usbdrvd.dll must be accessible to the DppApi in order to function properly.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

### DppApi Setup Notes

# Module PX4 Text Functions

Filename: DppApi.h

Description

Sets and reads text stored in the PX4.

PX4 Text Functions works for PX4 with FPGA/Firmware Version 3.13 and above.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

### PX4 Text Functions

GetPX4Text

SetPX4Text

# Module Status Indicator Functions

Filename: DppApi.h

Description

Status Indicator Functions monitor and display DPP device information.

Status information is read as a block of data and is converted into display values. Status information includes device identification, spectrum accumulation status, monitors, and boot status.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

### Status Indicator Functions

CreateMCAFileDPPSettings

DisplayStatusBuffer

GetStatusBuffer

GetStatusString

GetStatusStringFromBuffer

GetStatusStruct

GetStatusStructFromBuffer

ProcessStatusBuffer

# Module USB Manager Functions

Filename: DppApi.h

Description

USB Manager Functions provide USB device control.

USB Manager Functions provide USB device control.    All USB communications to the DPP require that the USB device be open for the corresponding DPP.    When a USB device is opened with OpenUSBDevice communications are opened to the DPP.    All USB functions are handled within the DppApi.

Copyright (c)2005, 2007 Amptek, All Rights Reserved

Developer Notes

Once all USB operations are completed and before the DppApi is closed the USB device should be closed with CloseUSBDevice.    The number of USB device connections can be monitored with MonitorUSBDppDevices.    If a device has been disconnected, MonitorUSBDppDevices will indicate the number of devices has decreased and appropriate action can be taken.

### USB Manager Functions

CloseUSBDevice

GetUSBDppDeviceInfo

MonitorUSBDppDevices

OpenUSBDevice

OpenUSBDeviceEx

# ClearDppData

void __stdcall ClearDppData(void * objptr, BOOLEAN isBufferA)

Clears the USB DPP spectrum data.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

isBufferA

Spectrum data buffer selector.

# CloseDppApi

void __stdcall CloseDppApi(void * objptr)

Closes the current instance of the DppApi.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

# CloseUSBDevice

void __stdcall CloseUSBDevice(void * objptr)

Closes the USB current device for communications.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

# CreateMCAFileDPPSettings

void __stdcall CreateMCAFileDPPSettings(void * objptr, LPWSTR pszSettings, LONG cSize)

Creates a MCA acquisition file string that includes all configuration and status settings.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszSettings

DPP devices settings.

cSize

Character buffer size.

Developer Notes

This function formats the additional information for MCA spectrum file storage.    This settings string is appended to the end of the DPP MCA spectrum file after the spectrum data section.    This information is used by the Amptek acquisition software.

# DisplayStatusBuffer

void __stdcall DisplayStatusBuffer(void * objptr, UCHAR Status[], LPWSTR pszStatus, LONG cSize)

Processes raw status data, stores the status data in the DppApi, and returns a status display string.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Status[]

Status buffer.

pszStatus

Status character string.

cSize

Character buffer size.

Developer Notes

The raw status data is read from the DPP by calling GetStatusBuffer. DisplayStatusBuffer is used to update the DppApi to the current DPP status and request status display information.

# Get80MHzMode

long __stdcall Get80MHzMode(void * objptr)

Gets FPGA 80MHz Clock Mode for DP5 only, returns true if is a DP5 and in 80MHz Mode.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Developer Notes

The FPGA 80MHz Clock Mode indicator is set when a DPP status command is issued and a DPP device is attached or when SetFPGAClockDefault sets this value.    The DP5 FPGA 80MHz Clock Mode must be set to correctly interpret DP5 configuration file settings.

# GetCoarseGainSpinFromText

void __stdcall GetCoarseGainSpinFromText(void * objptr, byte * Spin, LPWSTR pszCoarseGain)

Creates an up-down spin control value from a coarse gain display string.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Spin

Up-down spin control setting.

pszCoarseGain

Coarse gain value string.

# GetCoarseGainTextFromSpin

void __stdcall GetCoarseGainTextFromSpin(void * objptr, byte Spin, LPWSTR pszCoarseGain, LONG cSize)

Creates a coarse gain display string from a selected up-down spin control value.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Spin

Up-down spin control setting.

pszCoarseGain

Coarse gain value string.

cSize

Character buffer size.

# GetConfigFromBuffer

void __stdcall GetConfigFromBuffer(void * objptr)

Loads the stored configuration read from the DPP (stored in a byte block) into the DppApi.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi. configuration buffer read from the dpp stored in a byte block the current device type (default is 1=DP4)

# GetConfigFromDpp

void __stdcall GetConfigFromDpp(void * objptr)

Loads the stored configuration from the DPP USB into the DppApi.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

# GetConfigFromFile

void __stdcall GetConfigFromFile(void * objptr, LPWSTR pszFilename, LONG cSize, byte DPPDeviceType)

Loads a configuration from a DPP configuration file into the DppApi.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszFilename

Dpp configuration filename.

cSize

Character buffer size.

DPPDeviceType

DPP Device Type

Developer Notes

DPP Device Type (1=DP4,2=PX4,3=DP5(DP4 Emulation),4=DP5(Px4 Emulation)) If no device type or an invalid device type is selected, DPPDeviceType defaults to PX4.

# GetConfigString

void __stdcall GetConfigString(void * objptr, LPWSTR pszConfig, LONG cSize)

Creates display formatted copy of the current DppApi configuration.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszConfig

Configuration display string.

cSize

Character buffer size.

# GetDP5Text

void __stdcall GetDP5Text(void * objptr, LPWSTR pszDP5Text, LONG cSize)

Gets the DP5 stored text.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszDP5Text

DP5 text string.

cSize

Character buffer size.

# GetDppData

int __stdcall GetDppData(void * objptr, long DataBuffer[])

Get the USB DPP spectrum data.

Defined in: DppApi.h

Return Value

Returns the number of spectrum channels.

Parameters

objptr

Pointer to the DppApi.

DataBuffer[]

Holds the DPP spectrum data.

# GetOutputOffsetSpinFromText

void __stdcall GetOutputOffsetSpinFromText(void * objptr, byte * Spin, LPWSTR pszOffset)

Creates an up-down spin control value from an output offset display string.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Spin

Up-down spin control setting.

pszOffset

Output offset value string.

# GetOutputOffsetTextFromSpin

void __stdcall GetOutputOffsetTextFromSpin(void * objptr, byte Spin, LPWSTR pszOffset, LONG cSize)

Creates an output offset display string from a selected up-down spin control value.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Spin

Up-down spin control setting.

pszOffset

Output offset value string.

cSize

Character buffer size.

# GetPresetTimeStringFromVal

void __stdcall GetPresetTimeStringFromVal(void * objptr, long lPresetTime, LPWSTR pszPresetTime, LONG cSize)

Creates a preset time display string from a preset time value.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

lPresetTime

Preset time in tenths of a second.

pszPresetTime

Preset time display string.

cSize

Character buffer size.

# GetPresetTimeValFromString

void __stdcall GetPresetTimeValFromString(void * objptr, long * lPresetTime, LPWSTR pszPresetTime)

Calculates a preset time value from a preset time display string.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

lPresetTime

Preset time in tenths of a second.

pszPresetTime

Preset time display string.

# GetPX4Text

void __stdcall GetPX4Text(void * objptr, LPWSTR pszPX4Text, LONG cSize)

Gets the PX4 stored text.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszPX4Text

PX4 text string.

cSize

Character buffer size.

# GetSlowThreshSpinFromChannel

void __stdcall GetSlowThreshSpinFromChannel(void * objptr, double dblChannel, byte * Spin)

Calculates the up-down spin control value for a given slow threshold channel value.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

dblChannel

Slow threshold spectrum channel.

Spin

Up-down spin control setting.

# GetSlowThreshSpinFromPercent

void __stdcall GetSlowThreshSpinFromPercent(void * objptr, double dblPercent, byte * Spin)

Calculates the up-down spin control value for a given slow threshold percent value.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

dblPercent

Slow threshold percent of full energy scale.

Spin

Up-down spin control setting.

# GetSlowThreshStringsFromSpin

void __stdcall GetSlowThreshStringsFromSpin(void * objptr, byte Spin, LPWSTR pszPercent, LONG cSizePer, LPWSTR pszChannels, LONG cSizeChan)

Creates slow threshold percent and channel display strings from an up-down spin control value.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Spin

Up-down spin control setting.

pszPercent

Slow threshold percent display string.

cSizePer

Size of the percent character buffer.

pszChannels

Slow threshold channel position display string.

cSizeChan

Size of the channel character buffer.

# GetStatusBuffer

void __stdcall GetStatusBuffer(void * objptr, BOOLEAN isBufferA, UCHAR Status[])

Requests the current DPP device and spectrum status raw data.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

isBufferA

Spectrum data buffer selector.

Status[]

Status buffer.

Developer Notes

A spectrum buffer (A or B) is selected to determine which group of Spectrum Accumulation Status information to return.

# GetStatusString

void __stdcall GetStatusString(void * objptr, BOOLEAN isBufferA, LPWSTR pszStatus, LONG cSize)

Gets a copy of the current status and stores the data in a string.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

isBufferA

Spectrum data buffer selector.

pszStatus

Status character string.

cSize

Character buffer size.

Developer Notes

A spectrum buffer (A or B) is selected to determine which group of Spectrum Accumulation Status information to return.

# GetStatusStringFromBuffer

void __stdcall GetStatusStringFromBuffer(void * objptr, BOOLEAN isBufferA, LPWSTR pszStatus, LONG cSize, UCHAR Status[])

Gets a copy of the current status and stores the data in a string.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

isBufferA

Spectrum data buffer selector.

pszStatus

Status character string.

cSize

Character buffer size.

Status[]

Status buffer.

Developer Notes

A spectrum buffer (A or B) is selected to determine which group of Spectrum Accumulation Status information to return.

# GetStatusStruct

void __stdcall GetStatusStruct(void * objptr, BOOLEAN isBufferA, DPP_STATUS * StatusStruct)

Returns the current DPP device and spectrum status data in a status structure.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

isBufferA

Spectrum data buffer selector.

StatusStruct

Status structure.

Developer Notes

A spectrum buffer (A or B) is selected to determine which group of Spectrum Accumulation Status information to return.

# GetStatusStructFromBuffer

void __stdcall GetStatusStructFromBuffer(void * objptr, BOOLEAN isBufferA, DPP_STATUS * StatusStruct, UCHAR Status[])

Returns the current DPP device and spectrum status data in a status structure.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

isBufferA

Spectrum data buffer selector.

StatusStruct

Status structure.

Status[]

Status buffer.

Developer Notes

A spectrum buffer (A or B) is selected to determine which group of Spectrum Accumulation Status information to return.

# GetTempConfigSettings

void __stdcall GetTempConfigSettings(void * objptr, DPP_CONFIG_SETTINGS * CfgSet, BOOLEAN CurrentUpdate)

Copies selected configuration to a DPP_CONFIG_SETTINGS structure.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

CfgSet

Configuration settings.

CurrentUpdate

Current settings update.

Developer Notes

If UpdateCurrent is selected, the configuration is copied from the current DppApi configuration settings.    If UpdateCurrent is not selected, the last temporary configuration storage is used.

# GetTempConfigString

void __stdcall GetTempConfigString(void * objptr, LPWSTR pszConfig, LONG cSize)

Creates a display formatted copy of the temporary DppApi configuration.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszConfig

Configuration display string.

cSize

Character buffer size.

# GetTimeToPeakDecimationFromSpin

void __stdcall GetTimeToPeakDecimationFromSpin(void * objptr, byte * TimeToPeak, byte * Decimation, byte Spin)

Calculates the time-to-peak and decimation values from an up-down spin control value.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

TimeToPeak

Time-to-peak value.

Decimation

Decimation value.

Spin

Up-down spin control setting.

# GetTimeToPeakSpinFromValues

void __stdcall GetTimeToPeakSpinFromValues(void * objptr, byte TimeToPeak, byte Decimation, byte * Spin)

Converts a time-to-peak value to an up-down spin control setting.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

TimeToPeak

Time-to-peak value.

Decimation

Decimation value.

Spin

Up-down spin control setting.

Developer Notes

When a time-to-peak value is known it may be necessary to remotely set an up-down spin control.    GetTimeToPeakSpinFromValues uses a time-to-peak value a known decimation value to calculate the correct control setting.

# GetUSBDppDeviceInfo

int __stdcall GetUSBDppDeviceInfo(void * objptr, long Device, long * lSerialNumber)

Get device information (serial number and type) for the selected usb device.

Defined in: DppApi.h

Return Value

Returns the type Amptek USB device detected.

Amptek DPP USB devices

DPPNONE = 0x0		// none

DPPDP4 = 0x1			// DP4

DPPPX4 = 0x2			// PX4

DPPDP4EMUL = 0x3		// DP5 with DP4 Emulation

DPPPX4EMUL = 0x4		// DP5 with PX4 Emulation (Same as DPPDP5)

DPPDP5 = 0x4			// DP5 with PX4 Emulation

Parameters

objptr

Pointer to the DppApi.

Device

selected USB DPP device

lSerialNumber

serial number of device

# MonitorUSBDppDevices

int __stdcall MonitorUSBDppDevices(void * objptr)

Monitors the USB current device to determine if connected and communicating.

Defined in: DppApi.h

Return Value

Returns the number of Amptek USB devices detected.

Parameters

objptr

Pointer to the DppApi.

# OpenDppApi

void * __stdcall OpenDppApi(void)

Creates and initializes an instance of the DppApi.

Defined in: DppApi.h

Return Value

Returns an object pointer handle to the DppApi.

# OpenUSBDevice

int __stdcall OpenUSBDevice(void * objptr)

Opens the USB current device for communications.

Defined in: DppApi.h

Return Value

Returns the number of Amptek USB devices detected.

Parameters

objptr

Pointer to the DppApi.

# OpenUSBDeviceEx

int __stdcall OpenUSBDeviceEx(void * objptr, long Device)

Opens the USB selected device for communications.

Defined in: DppApi.h

Return Value

Returns the number of Amptek USB devices detected.

Parameters

objptr

Pointer to the DppApi.

Device

selected USB DPP device

# PauseDppData

void __stdcall PauseDppData(void * objptr, BOOLEAN boolPauseData)

Pauses the USB DPP spectrum data.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

boolPauseData

Pause the spectrum data.

# ProcessDppData

int __stdcall ProcessDppData(void * objptr, long DataBuffer[], UCHAR RawData[], UINT ByteCount)

Process the USB DPP spectrum data.

Defined in: DppApi.h

Return Value

Returns the number of spectrum channels.

Parameters

objptr

Pointer to the DppApi.

DataBuffer[]

Holds the DPP spectrum data.

RawData[]

Holds pre-processed spectra data.

ByteCount

Holds pre-processed spectra data byte count.

# ProcessStatusBuffer

void __stdcall ProcessStatusBuffer(void * objptr, UCHAR Status[])

Processes raw status data and stores the status data in the DppApi.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Status[]

Status buffer.

Developer Notes

The raw status data is read from the DPP by calling GetStatusBuffer. ProcessStatusBuffer is used to update the DppApi to the current DPP status without requesting status display information.

# SaveConfigToFile

void __stdcall SaveConfigToFile(void * objptr, LPWSTR pszFilename, LONG cSize)

Sends the current configuration from the DppApi to a DPP configuration file.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszFilename

Dpp configuration filename.

cSize

Character buffer size.

# SendConfigToBuffer

void __stdcall SendConfigToBuffer(void * objptr)

Sends the current configuration from the DppApi to a configuration buffer.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi. dppapi configuration stored in a byte block preset setting

# SendConfigToDpp

void __stdcall SendConfigToDpp(void * objptr)

Sends the current configuration from the DppApi to the DPP USB.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

# SendDppVendorRequest

void __stdcall SendDppVendorRequest(void * objptr, int Request, int Value)

Sends a generic DPP Vendor Request to the default usb device.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Request

Vendor request.

Value

Vendor request value.

Developer Notes

The function SendDppVendorRequest is determined by the Request parameter. See DP4/PX4/DP5 user manuals for Vendor Request function details.

Boot Option Vendor Requests:

BootOptSetBootFlags = 0x92                      //Set DP5 boot options

BootOptSetPC5HV = 0x93                              //Set PC5 HV

BootOptPC5TECTemp = 0x94                          //Set PC5 TEC temperature

BootOptDP5InputOffset = 0x95                  //Set DP5 input offset

BootOptSetDP5uCtrADC = 0x96                    //Set DP5 microcontroller ADC/Temperature calibration (RESERVED)

BootOptSetDP5SpectrumOffset = 0x97      //Set DP5 spectrum offset

# SetDP5Text

void __stdcall SetDP5Text(void * objptr, LPWSTR pszDP5Text)

Sets the DP5 stored text.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszDP5Text

DP5 text string.

# SetFPGAClockDefault

void __stdcall SetFPGAClockDefault(void * objptr, BOOLEAN b80MHzMode, byte DPPDeviceType)

Sets FPGA 80MHz Clock Mode indicator for DP5 only

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

b80MHzMode

80MHz Mode indicator.

DPPDeviceType

DPP Device Type.

Developer Notes

The DP5 FPGA 80MHz Clock Mode indicator must be set to correctly interpret DP5 configuration file settings.    The FPGA 80MHz Clock indicator is set when a DPP status command is issued and a DPP device is attached or when SetFPGAClockDefault sets this value.    SetFPGAClockDefault is needed when editing DP5 configuration files without a DP5 device.

# SetPX4Text

void __stdcall SetPX4Text(void * objptr, LPWSTR pszPX4Text)

Sets the PX4 stored text.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

pszPX4Text

PX4 text string.

# SetTempConfigSettings

void __stdcall SetTempConfigSettings(void * objptr, DPP_CONFIG_SETTINGS * CfgSet, BOOLEAN CurrentUpdate)

Copies the selected settings from a DPP_CONFIG_SETTINGS data structure.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

CfgSet

Configuration settings.

CurrentUpdate

Current settings update.

Developer Notes

If UpdateCurrent is selected, the temporary configuration is also copied to the current DppApi configuration settings.    Current DppApi configuration settings can be used to configure the DPP or be saved to a configuration file for storage.

# TuneFastThreshold

void __stdcall TuneFastThreshold(void * objptr)

Tunes the fast threshold.

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Developer Notes

The source must be removed before tuning the fast threshold.

The status flag SetFastThreshDone is set (=TRUE) if the fast threshold tuning was successful.    If the fast threshold tuning was successful, the configuration must be read back using GetConfigFromDpp to update the DppApi copy of the configuration.

# TuneInputOffset

void __stdcall TuneInputOffset(void * objptr)

Tunes the input offset. (PX4/DP5 only.)

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

Developer Notes

Input Offset MUST be properly set before running TuneInputOffset. The initial input offset setting is based on the current input polarity and polezero configuration settings.    The TuneInputOffsetInit function checks these values and sets the initial input offset value.

The status flag SetInputOffsetDone is set (=TRUE) if the input offset tuning was successful.    If the input offset tuning was successful, the configuration must be read back using GetConfigFromDpp to update the DppApi copy of the configuration.

(See the DppApi Reference and the PX4/DP5 Hardware Description for details.)

Possible input offset tune start settings:

SET TO -2.048V (= 0000) IF THE Input Polarity is NEGATIVE(-) and PoleZero > 0

SET TO -0.100V (= 1948) IF THE Input Polarity is NEGATIVE(-) and PoleZero = 0

SET TO +0.100V (= 2148) IF THE Input Polarity is POSITIVE(+) and PoleZero = 0

SET TO +2.042V (= 4090) IF THE Input Polarity is POSITIVE(+) and PoleZero > 0

# TuneInputOffsetInit

void __stdcall TuneInputOffsetInit(void * objptr)

Sets initial values for PX4 input offset tuning. (PX4 only.)

Defined in: DppApi.h

Parameters

objptr

Pointer to the DppApi.

# DPP_BOOT_STATUS Structure

typedef struct {

USHORT BootFlags;

USHORT HVDACSet;

USHORT TECDACSet;

USHORT InputOffsetTuning;

USHORT uCTempCal;

USHORT SpectrumOffset;

} DPP_BOOT_STATUS;

DPP Boot Options Status Storage Type

Defined in: DppApi.h

Members

BootFlags

Boot flags (MSB currently unused)

HVDACSet

HV DAC setting (not used in PX4 mode)

TECDACSet

TEC DAC setting (not used in PX4 mode)

InputOffsetTuning

Input offset DAC setting (not used in PX4 mode)

uCTempCal

uC temp cal

SpectrumOffset

Spectrum offset

# DPP_CONFIG_SETTINGS Structure

typedef struct {

byte AcqMode;

byte MCSTimebase;

byte MCAChannels;

byte BufferSelect;

byte TTLGate;

byte SlowThreshold;

long PresetTime;

long PresetCount;

long SCA[8];

byte TimeToPeak;

byte Decimation;

byte FlatTop;

byte PUREnable;

byte FastThreshold;

byte RTDSlow;

byte RTDOn;

byte RTDFast;

byte BLR;

byte BaselineOn;

byte CoarseGain;

long FineGain;

long PoleZero;

long InputOffset;

byte InputPolarity;

byte DetReset;

long TEC;

byte HVEnabled;

long HV;

byte PreampPower;

byte AnalogOut;

byte OutputOffset;

byte AuxOut;

byte AudibleCounter;

} DPP_CONFIG_SETTINGS;

DPP Configuration Settings Storage Type

Defined in: DppApi.h

Members

AcqMode

acquisition mode 0=MCA,1=MCS

MCSTimebase

MCS timebase value (0-15) see CAcqMode

MCAChannels

number of channels 4=256,3=512,2=1024,1=2048,0=4096,5=8192

BufferSelect

Holds Buffer Sel A&B,Buffer Sel Hardware,see DPPBufferSelect

TTLGate

gate input settings, determines events included/excluded from spectrum, see DPPGate

SlowThreshold

Slow ch threshold, Events w/amp lower not added to spectrum

PresetTime

var holds preset time, used in usb

PresetCount

preset count in selected channels, ch are set in SCA8

SCA[8]

SCA values, LL, UL, and Enable are stored together

TimeToPeak

TimeToPeak register setting

Decimation

decimation setting for pulse shaping

FlatTop

flatop register setting

PUREnable

pile-up rejection enabled

FastThreshold

Fast Ch Threshold, events w/fch amp below this are rejected

RTDSlow

Risetime Discrimination slow threshold

RTDOn

Turns RTD on, and sets the amplitude and timing thresholds

RTDFast

RTD Time Threshold,Events w/HWHM wider than this are rejected

BLR

Baseline Restoration, see udBLR.Value notes for values

BaselineOn

use autobaseline during detector reset

CoarseGain

stores current coarse gain value

FineGain

stores current fine gain value

PoleZero

pole zero adjust value

InputOffset

input offset

InputPolarity

use InvertEnable during detector reset

DetReset

detector reset lockout period

TEC

TEC temperature setting (displayed in Kelvin)

HVEnabled

high voltage setting enable

HV

high voltage setting value

PreampPower

preamp power select value (5v or 8.5v)

AnalogOut

dac enabled and DAC output type,(stobed peak,shaped pulse,dec inp,fast ch)

OutputOffset

Output DAC offset, -64…+63,(signed)(D7-D1) (-500mV to +492mV)

AuxOut

Aux output type

AudibleCounter

audio volume setting

# DPP_STATUS Structure

typedef struct {

double FPGA;

double Firmware;

double SerialNumber;

byte StatDevInd;

byte BootStatus;

byte PwrBtnConfig;

byte SwConfigRcvd;

byte SetFastThreshDone;

byte SetSlowThreshDone;

byte SetInputOffsetDone;

double BoardTemp;

double HVMonitor;

double TECMonitor;

double FastCount;

double SlowCount;

double AccumulationTime;

byte StatMcaEnabled;

byte MCSDone;

byte PresetCountExpired;

} DPP_STATUS;

DPP Status Storage Type

Defined in: DppApi.h

Members

FPGA

FPGA

Firmware

firmware revision

SerialNumber

unit serial number

StatDevInd

device indicator from status block (0=dp4,1=px4)

BootStatus

boot status byte (PwrBtnConfig,SwConfigRcvd,McaEnabled)

PwrBtnConfig

dpp has loaded config from power button

SwConfigRcvd

software has sent valid config to unit since startup

SetFastThreshDone

DPP tune fast thresh has successfully done

SetSlowThreshDone

DPP tune slow thresh has successfully done (NOT USED)

SetInputOffsetDone

DPP tune input offset has successfully done

BoardTemp

board temperature monitor value

HVMonitor

high voltage monitor value

TECMonitor

TEC temperature monitor value (displayed in Kelvin)

FastCount

fast channel count

SlowCount

slow channel count

AccumulationTime

real time duration of present data acquisition interval

StatMcaEnabled

mca enabled status, high during acq, preset time resets flag

MCSDone

MCS done flag 1=finished, 0=not finished

PresetCountExpired

indicates presets counts have been reached