This document describes the build procedure used for software.

Modify the file controller/version.h to contain the latest rev ID and build number.

Update hostdown.ini to include an auto-download set that can download all the necessary configuration files.

Setup ClearCase for the build

- In ClearCase, using the appropriate view (i.e. rel1_build), create a public label (of the

form of “BLD_J.024”) for the new release.

- In ClearCase, make sure there are no checkouts.

- In ClearCase, using the appropriate view (i.e. rel1_build), label all elements.

Create a build directory on the VIP-1 server in the directory Vip-9_Software/”release”.

Within the build directory, create the following directories

command_processor

configure

vip_comm

viva

The following assumes the ClearCase view created above is mounted on the Z: drive

system_software.bin:

- build using Tornado 1.0.1 with GNU Toolkit 2.7 and:

Build Target as "TGT_DIR=/vxworks/target Vip_FLASH"

working directory as "z:/cmd_proc/controller/build"

- copy z:/cmd_proc/controller/build/system_software.bin to

build directory/command_processor.

hostdown application and ini file:

- copy hostdown.exe from the previous build to the build directory/command_processor

- copy z:/cmd_proc/host_down/hostdown.ini to build directory/command_processor

Communications DLL:

- open Visual C++ 6.0

- from the Visual C++ 6.0 environment, open z:/cmd_proc/comm_dll/vip_comm.dsw

- select Build:Rebuild All from menu (release version)

- copy z:/cmd_proc/comm_dll/release/vip_comm.dll and

vip_comm.lib to build directory/vip_comm

- copy z:/cmd_proc/comm_dll/vip_comm.h to

to build directory/vip_comm

- copy z:/cmd_proc/comm_dll/communications_interface_design.doc to build

directory/vip_comm

- copy z:/cmd_proc/comm_dll/sio_cmds_usage.doc to build directory/vip_comm

Configure application:

- open Visual C++ 6.0

- from the Visual C++ 6.0 environment, open z:/cmd_proc /config_app/configure.dsw

- select Build:Rebuild All from menu (release version)

- copy z:/cmd_proc/config_app/release/configure.exe to build directory/configure

- copy z:/cmd_proc/config_app/cp30fwm.dll to build directory/configure

- copy z:/cmd_proc/config_app/ss32d25.dll to build directory/configure

- copy z:/cmd_proc/comm._dll/vip_comm..dll to build directory/configure

ViVA application:

- open Visual C++ 6.0

- from the Visual C++ 6.0 environment, open z:/cmd_proc/viva/ic.dsw

- select Build:Rebuild All from menu (release version)

- copy z:/cmd_proc/viva/release/viva.exe to build directory/viva

- copy z:/cmd_proc/comm_dll/release/vip_comm.dll to build directory/viva

- copy z:/cmd_proc/viva/viva_help.doc to build directory/viva

- copy z:/cmd_proc/viva/wd97vwr32.exe to build directory/viva

vip_prom.hex

- build using Tornado 1.0.1 with GNU Toolkit 2.7 and:

Build Target as "TGT_DIR=/vxworks/target Vip_PROM"

working directory as "z:/cmd_proc/controller/build"

- copy z:/vxworks/target/config/vipgi/vxWorks.cmp_rom.hex to

build directory/command_processor/vip_prom.hex.

vxw_prom.hex

- build using Tornado 1.0.1 with GNU Toolkit 2.7 and:

Build Target as “TGT_DIR=/vxworks/target Vxorks_PROM”

Working directory as “z:/cmd_proc/controller/build”

- copy z:/vxworks/target/config/vipgi/vxworks_prom.hex to

build directory/command_processor/vxw_prom.hex

abs_firmware.bin

- verify no new release from vendor

- if no new release

- copy from previous build directory

- if there is a new release

- obtain new version from vendor and place in build directory

global_control_firmware.hex

- verify if the version has changed by consulting the hardware department

- if the version has not changed

- copy from previous build directory

- if the version has changed

- obtain latest version from the hardware department

- obtain CD containing files used to create firmware hex file.

- verify that files on CD are consistent with the date of the firmware hex file.

- execute the config_app program built in previous steps

- select “Engineering”

- select “Part #421030-01”

- under “Firmware Configuration”

- Enter Version name

- Select “Open”

- Select the appropriate global control firmware file

- copy the global control firmware file from the previous step into build

directory/command_processor/global_control_firmware.hex

std_receptor_firmware.hex

- verify if the version has changed by consulting the hardware department

- if the version has not changed

- copy from previous build directory

- if the version has changed

- obtain latest version from the hardware department

- obtain CD containing files used to create firmware hex file.

- verify that files on CD are consistent with the date of the firmware hex file.

- execute the config_app program built in previous steps

- select “Engineering”

- select “Part #421030-01”

- under “Firmware Configuration”

- Enter Version name

- Select “Open”

- Select the appropriate standard receptor firmware file

- copy the standard receptor firmware file from the previous step into build

directory/command_processor/std_receptor_firmware.hex

dcds_receptor_firmware.hex

- verify if the version has changed by consulting the hardware department

- if the version has not changed

- copy from previous build directory

- if the version has changed

- obtain latest version from the hardware department

- execute the config_app program built in previous steps

- select “Engineering”

- select “Part #421030-01”

- under “Firmware Configuration”

- Enter Version name

- Select “Open”

- Select the appropriate dcds receptor firmware file

- copy the dcds receptor firmware file from the previous step into build

directory/command_processor/dcds_receptor_firmware.hex

setup.exe

- copy from previous build directory

Update Software_Package.doc to reflect all new features added and bugs repaired and copy from z:/deployment/releases/software_package.doc to build directory.

Update build_procedure.doc to reflect all relevant changes and copy from z:/deployment/releases/build_procedure.doc.

Create software_package.zip file in the build directory by zipping build directory utilizing pkzip25.exe.  From build directory enter: Pkzip25 –add –Path –Recurse software_package.zip *.*

Copy all files from build directory to an archive CD (2) and check into ClearCase then update the label in ClearCase.

Update ECO.

Ship one CD and archive second CD.