AMD Socket S1g1 Motherboard Schematic Checklist Motherboard Vendor:. Board Name:. Schematic Revision Number:. Board Revision Number:. Processor:. Tunnel:. I/O Hub:. Reviewers:. Initial Date of Review:.

|  | Publication # |  | 33363 | Revision: |  | 1.09 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Issue Date: | May 2006 |  |  |  |  |  |  |  |  |  |  |

|  | © 2005, 2006 Advanced Micro Devices, Inc. All rights reserved. |  |
| --- | --- | --- |
|  | The contents of this document are provided in connection with Advanced Micro Devices, Inc. (“AMD”) products. AMD makes no representations or warranties with respect to the accuracy or completeness of the contents of this publication and reserves the right to make changes to specifications and product descriptions at any time without notice. The information contained herein may be of a preliminary or advance nature and is subject to change without notice. No license, whether express, implied, arising by estoppel, or otherwise, to any intellectual property rights are granted by this publication. Except as set forth in AMD’s Standard Terms and Conditions of Sale, AMD assumes no liability whatsoever, and disclaims any express or implied warranty, relating to its products including, but not limited to, the implied warranty of merchantability, fitness for a particular purpose, or infringement of any intellectual property right. AMD’s products are not designed, intended, authorized or warranted for use as components in systems intended for surgical implant into the body, or in other applications intended to support or sustain life, or in any other application in which the failure of AMD’s product could create a situation where personal injury, death, or severe property or environmental damage may occur. AMD reserves the right to discontinue or make changes to its products at any time without notice. |  |
| Trademarks |  |  |
| AMD, the AMD Arrow logo, AMD Athlon, and combinations thereof, are trademarks of Advanced Micro Devices, Inc. HyperTransport is a licensed trademark of the HyperTransport Technology Consortium. Other product names used in this publication are for identification purposes only and may be trademarks of their respective companies. |  |  |

Contents

Revision History	5

Introduction		6

Chapter 1	Clock Signals	7

Chapter 2	Two SO-DIMM DDR2 Schematic Checklist	8

Chapter 3	Miscellaneous Signals	10

Chapter 4	Power and Decoupling Requirements	12

Chapter 5	EMI Schematic Checklist	14

Chapter 6	HyperTransport™ Schematic Checklist	15

List of Tables

Table 1. Clock Signals	7

Table 2. Two SO-DIMM DDR2	8

Table 3. Miscellaneous Signals	10

Table 4. Power and Decoupling Requirements	12

Table 5. EMI Schematic Checklist	14

Table 6. HyperTransport™ Schematic Checklist	15

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| May 2006 | 1.09 | Accepted all changes |
| April 2006 | 1.08 | Added the following: - Item 3-39 Revised the following - Item 3-17, Item 3-18, and Item 3-21 |
| March 2006 | 1.07 | Accepted all changes |
| February 2006 | 1.06 | Added the following: - Item 2-9 - Item 3-16 - Items 4-28 and 4-29 Revised the following: - Item 3-22 - Items 4-4, 4-11, 4-13—4-15, and 4-22 |
| September 2005 | 1.05 | Changes accepted and change bars removed |
| September 2005 | 1.04 | Revised the following: - Items 2-1 and 2-12 - Items 3-2, 3-6, 3-18, 3-20, 3-21, and 3-38 - Items 4-4, 4-14, 4-19, and 4-26 |
| July 2005 | 1.03 | Changes accepted and change bars removed |
| July 2005 | 1.02 | Added the following: - Items 3-22—3-24 Revised the following: - Item 1-3 - Item 1-4 - Item 3-25 - Item 3-26 - Item 3-29 |
| June 2005 | 1.01 | Initial NDA release |
| June 2005 | 1.00 | Initial NDA draft |

# Introduction

NPT, or New Platform Technology, refers to the technology developed to support AMD processors in new packages, such as the AMD S1g1 processor.

This checklist accompanies the AMD S1g1 Processor Motherboard Design Guide, order# 33164, and is designed to help users evaluate whether the schematic design for miscellaneous, DDR, and power signals schematic conforms to the AMD minimum requirements. Check the schematic against the corresponding design rule in the checklist. In the Yes/No (Y/N) column, record whether or not the schematic follows the design rule. If an item cannot be marked Yes or No, mark the Y/N column with a V to indicate that the board designer must check the item for compliance. After completing the checklist, correct all items marked with an N to ensure that they match the design guide mentioned above.

If a discrepancy exists, a comment must be placed in the cell of the checklist item with the problem. Violations and issues that need attention or verification are color-coded red. Items that are acceptable or have been corrected are color-coded in blue. All comments must be in italic and bold typeface.

This checklist is not intended to identify all elements key to developing a motherboard, and AMD does not claim that compliance with this document ensures either compliance with standard practices in the industry or motherboard functionality. The manufacturer is solely responsible for ensuring that designs are fully functional and comply with all applicable industry standards and practices.

Failure to meet all design guide requirements found in this checklist will jeopardize the ability of the designed system to function in a reliable manner and could damage the processor, motherboard, or both. Because of this danger, AMD reserves the right to reject the design for submittal into any program that supports, validates, or recommends the motherboard in question.

# Clock Signals

Table 1. Clock Signals

| Item No. | Schematic | Explanation | Y/N |
| --- | --- | --- | --- |
| 1-1 | Clock Generator | Clocks should be driven by a 200-MHz differential clock source. |  |
| 1-2 |  | Two 3900 pF (10%) AC coupling capacitors are placed between the clock generator output and the processor clock input pins (CLKIN_H and CLKIN_L). |  |
| 1-3 |  | The clock generator is terminated with series 47.5- (1%) resistors that are placed on both the true and complement signals of the CLK net. |  |
| 1-4 |  | There is a 261- (1%) shunt resistor between the true and complement clock pins of the clock generator. This resistor should be between the series resistors at the clock generator and the AC coupling capacitors. |  |
| 1-5 |  | There is a 169- (1%) shunt resistor between the processor’s CLKIN_L and CLKIN_H pins. This resistor should be between the AC coupling capacitors and the processor CLKIN pins. |  |

# Two SO-DIMM DDR2 Schematic Checklist

Table 2. Two SO-DIMM DDR2

| Item No. | Schematic | Explanation | Y/N |
| --- | --- | --- | --- |
| 2-1 | Clock Termination | There is a 1.5-pF (± 10%) differential termination capacitor between CLK_H and CLK_L on all the memory clock nets. |  |
| 2-2 | No Termination for Data | The data nets (DATA, DQS, and DM) do not need onboard termination. |  |
| 2-3 | Connections | The following signals are connected to DIMM_A0: - MA_DATA[63:0] - MA_DQS_H[7:0] - MA_DQS_L[7:0] - MA_DM[7:0] - MA_ADD[15:0] - MA_BANK[2:0] - MA_RAS_L - MA_CAS_L - MA_WE_L |  |
| 2-4 |  | The following signals are connected to DIMM_B0: - MB_DATA[63:0] - MB_DQS_H[7:0] - MB_DQS_L[7:0] - MB_DM[7:0] - MB_ADD[15:0] - MB_BANK[2:0] - MB_RAS_L - MB_CAS_L - MB_WE_L |  |
| 2-5 |  | The Clock assignments to each DIMM are as follows: - MA0_CLK_H[2:1] and MA0_CLK_L[2:1] are connected to DIMM_A0. - MB0_CLK_H[2:1] and MB0_CLK_L[2:1] are connected to DIMM_B0. |  |
| 2-6 |  | The Chip Select (CS) assignments to each DIMM are as follows: - MA0_CS_L[3:0] is connected to DIMM_A0. - MB0_CS_L[3:0] is connected to DIMM_B0. |  |
| 2-7 |  | The CKE assignments to each DIMM are as follows: - MA_CKE[1:0] is connected to DIMM_A0. - MB_CKE[1:0] is connected to DIMM_B0. |  |
| 2-8 |  | The ODT assignments to each DIMM are as follows: - MA0_ODT[1:0] is connected to DIMM A0. - MB0_ODT[1:0] is connected to DIMM B0. |  |
| 2-9 |  | The VDDSPD pin on each SO-DIMM pin is powered by 3.3 V. |  |
| 2-10 | Address and Command Termination | The following signals have RTT termination of 47  to VTT: - MA_ADD[15:0], MB_ADD[15:0] - MA_BANK[2:0], MB_BANK[2:0] - MA_RAS_L, MB_RAS_L - MA_CAS_L, MB_CAS_L - MA_WE_L, MB_WE_L |  |
| 2-11 | CKE, CS and ODT net Termination | MA_CKE[1:0] and MB_CKE[1:0] nets have RTT termination of 47  to VTT. |  |
| 2-12 |  | MA0_CS_L[3:0] and MB0_CS_L[3:0] nets have RTT termination of 47  to VTT. |  |
| 2-13 |  | All the ODT nets have RTT termination of 47  to VTT. |  |

# Miscellaneous Signals

Table 3. Miscellaneous Signals

| Item No. | Schematic | Explanation | Y/N |
| --- | --- | --- | --- |
| 3-1 | Voltage Tolerance | All miscellaneous input pins should have voltage translation to VDDIO level. |  |
| 3-2 | Power Supply Related Nets | The VID[5:0] bits are routed to the VDD power supply. A 6-bit controller is required. |  |
| 3-3 |  | VID[1] should have a 300- pullup to VDDIO |  |
| 3-4 |  | VDD_FB_L and VDD_FB_H are routed to the VDD power supply. |  |
| 3-5 |  | VDDIO_FB_L and VDDIO_FB_H are routed to the VDDIO power supply. |  |
| 3-6 |  | VTT_SENSE is routed to the VTT regulator. Note: VTT_SENSE can be left unconnected in cases where the VTT regulator does not have a feedback connection. |  |
| 3-7 |  | PSI_L is routed to the power supply control circuit. |  |
| 3-8 | Drive Strength Setting | HTREF1 is pulled up to VLDT with a 44.2- resistor (1%). |  |
| 3-9 |  | HTREF0 is pulled down to VSS with a 44.2-resistor (1%). |  |
| 3-10 |  | M_ZN is pulled up to VDDIO with a 39.2-resistor (1%). |  |
| 3-11 |  | M_ZP is pulled down to VSS with a 39.2-resistor (1%). |  |
| 3-12 | Miscellaneous HyperTransport Nets | PWROK is routed to the PWROK generation circuit. |  |
| 3-13 |  | RESET_L is routed to the RESET_L source (I/O Hub). |  |
| 3-14 |  | LDTSTOP_L is routed to the I/O Hub. |  |
| 3-15 |  | If LDTSTOP_L, RESET_L, or PWROK are driven by an open-drain driver, then each one that is driven by an open-drain driver needs to have a 300-(5%) pullup to VDDIO. |  |
| 3-16 |  | The design needs to have circuitry to help prevent glitches on LDTSTOP/RST/PWROK upon entry to and exit from S3. Please refer to the appropriate vendor chipset documentation. |  |
| 3-17 | Thermal Monitoring | If PROCHOT_L input throttling is implemented, PROCHOT_L is routed to the thermal monitoring circuit. |  |
| 3-18 |  | PROCHOT_L has a 300- (5%) pullup to VDDIO, regardless of whether PROCHOT_L input throttling is implemented or not. |  |
| 3-19 |  | THERMTRIP_L is routed to the thermal monitoring circuit and powers off the processor within the THERMTRIP_L power-down time limit. |  |
| 3-20 |  | THERMTRIP _L has a 300- (5%) pullup to VDDIO. |  |
| 3-21 |  | The SID pin can be left unconnected. The SIC pin should strapped to either VDDIO or VSS (but not both) using a 300-Ω (±5%) pull-down resistor. |  |
| 3-22 |  | THERMDA is connected to the positive side of an analog input of a temperature-monitoring device. THERMDA should only be connected to one device. |  |
| 3-23 |  | THERMDC is connected to the negative side of an analog input of a temperature-monitoring device. THERMDC should only be connected to one device. |  |
| 3-24 |  | THERMDA and THERMDC are connected to a dual-sourcing current temperature sensor. Neither signal is connected to a single or triple (or higher) sourcing current temperature sensor. |  |
| 3-25 | TEST and RSVD pins | These pins have no onboard connection: TEST2, TEST3, TEST6, TEST7, TEST8, TEST10, TEST26, TEST27, TEST28_H, TEST28_L, and any RSVD pin |  |
| 3-26 |  | These pins have onboard test points: TEST12, TEST14, TEST15, TEST16, TEST17, TEST18, TEST19, TEST20, TEST21, TEST22, TEST23, and TEST24 |  |
| 3-27 |  | TEST25_H has 510- pullup to VDDIO. |  |
| 3-28 |  | TEST25_L has 510- pulldown to VSS. |  |
| 3-29 |  | TEST9 and TEST13 are tied to VSS. |  |
| 3-30 |  | There is an 80.6- (1%) differential termination between TEST29_L and TEST29_H. |  |
| 3-31 | HDT Header Connections | TMS is routed to pin 13 of the HDT header. |  |
| 3-32 |  | TCK is routed to pin 11 of the HDT header. |  |
| 3-33 |  | TRST_L is routed to pin 17 of the HDT header. |  |
| 3-34 |  | TDI is routed to pin 15 of the HDT header. |  |
| 3-35 |  | TDO is routed to pin 19 of the HDT header. |  |
| 3-36 |  | DBREQ_L is routed to pin 7 of the HDT header. |  |
| 3-37 |  | DBRDY is routed to pin 9 of the HDT header. |  |
| 3-38 |  | RESET_L is level shifted to 3.3 V and routed to pin 24 of the HDT header. |  |
| 3-39 | Warm Reset | A warm reset switch physically mounted to the motherboard must be provided to enable AMD debug capabilities. This switch can be a momentary switch connecting the processor HyperTransport™ reset signal to VSS. In production, this switch can be no-popped, but the site must never be removed. This switch functionality cannot be wired to a chassis-mounted switch. |  |

# Power and Decoupling Requirements

Table 4. Power and Decoupling Requirements

| Item No. | Schematic | Explanation | Y/N |
| --- | --- | --- | --- |
| 4-1 | VDDIO to VSS Decoupling | Two 22-F ceramic capacitors in 1206 or 0805 packages with X7R dielectric |  |
| 4-2 |  | Six 0.22-F ceramic capacitors in 0603 packages with X7R dielectric |  |
| 4-3 |  | Two 0.01-F ceramic capacitors in 0603 packages with X7R dielectric |  |
| 4-4 |  | One 180-pF ceramic NPO or COG capacitor in 0402 packages Note: If the distance between the processor’s keepout and the first DIMM’s keepout is less than 2.5 inches, there should be two additional 180-pF capacitors. If the distance between the processor’s keepout and the first DIMM’s keepout is greater than 2.5 inches, there should be four additional 180-pF capacitors. |  |
| 4-5 |  | Four 4.7-F ceramic capacitors in 0805 or 0603 packages with X7R dielectric |  |
| 4-6 | VTT Decoupling | VTT is decoupled to VSS with four 4.7-F ceramic capacitors in 0805 packages with X7R dielectric. |  |
| 4-7 |  | VTT is decoupled to VSS with four 0.22-F ceramic capacitors in 0603 packages with X7R dielectric. |  |
| 4-8 |  | VTT is decoupled to VSS with four 1-nF ceramic capacitors in 0603 or 0402 packages with X7R dielectric. |  |
| 4-9 |  | VTT is decoupled to VSS with one 0.1-F ceramic capacitor in a 0603 package with X7R dielectric for each RTT Rpack (Address/Command termination resistors to VTT). |  |
| 4-10 |  | VTT is decoupled to VDDIO with one 0.1-F ceramic capacitor in a 0603 package with X7R dielectric for each RTT Rpack (Address/Command termination resistors to VTT). |  |
| 4-11 |  | VTT is decoupled to VSS with four 180-pF ceramic COG or NPO capacitors in 0402 packages. |  |
| 4-12 |  | There are sufficient bulk capacitors, as specified by the power supply design. |  |
| 4-13 | M_VREF | The VREF source is not shared between the DIMMs and the processor. |  |
| 4-14 |  | M_VREF should have one 0.1-F and one 1-nF capacitor for decoupling to VSS. |  |
| 4-15 |  | The DIMM VREF has decoupling capacitance. |  |
| 4-16 | VDD to VSS Decoupling | Nine 22-µF ceramic capacitors in 1206 or 0805 packages with X7R dielectric |  |
| 4-17 |  | Two 0.22-µF ceramic capacitors in 0603 packages with X7R dielectric |  |
| 4-18 |  | One 0.01-µF ceramic capacitor in a 0603 package with X7R dielectric |  |
| 4-19 |  | One 180-pF COG or NPO ceramic capacitor in a 0402 package |  |
| 4-20 |  | There are sufficient bulk capacitors, as specified by the power supply design. |  |
| 4-21 | VDDA Circuit and Filter | There is a 100-F capacitor at the output of the VDDA supply. |  |
| 4-22 |  | VDDA is filtered with a ferrite bead with an approximate impedance of 35 , a maximum DC resistance of 40 m, and a current rating of at least 500 mA. |  |
| 4-23 |  | There is one 0.22-F, one 3300-pF, and one 4.7-F capacitor placed on VDDA. |  |
| 4-24 | VLDT to VSS Decoupling | Three 4.7-µF ceramic capacitors in 0805 packages with X7R dielectric |  |
| 4-25 |  | Two 0.22-µF ceramic capacitors in 0603 packages with X7R dielectric |  |
| 4-26 |  | Two 180-pF ceramic COG or NPO capacitors in 0402 packages |  |
| 4-27 |  | There are sufficient bulk capacitors, as specified by the power supply design |  |
| 4-28 | S3 Power Supplies | During the S3 power state entry or exit, all processor power supplies (VTT, VDDIO, and VLDT) meet the sequencing requirements that are specified in AMD NPT Family 0Fh Processor Electrical Data Sheet, order# 31119 |  |
| 4-29 | SO-DIMM Implementations | The SO-DIMM’s VDDSPD input is powered from of a 3.3-V supply. |  |

# EMI Schematic Checklist

Table 5. EMI Schematic Checklist

| Item No. | Schematic | Explanation | Y/N |
| --- | --- | --- | --- |
| 5-1 | EMI | The clock generator is capable of being set to 0.5 % down spread. |  |
| 5-2 |  | All high-speed (33 MHz or greater), single-ended clock signals contain a series-terminating resistor. |  |
| 5-3 |  | All motherboard-mounting holes should connect to ground, either directly or through two parallel zero-Ω resistors. |  |

# HyperTransport™ Schematic Checklist

Table 6. HyperTransport™ Schematic Checklist

| Item No. | Schematic | Description | Y/N |
| --- | --- | --- | --- |
| 6-1 | Special Case Link Termination | If a 16 x 16 link is connected to another 16 x 16 device, all CAD and CLK nets are point-to-point. CTLIN[1] is terminated to logic high with 51- (± 5%) resistors. Note: For differential logic high, pull the true signal high and the complement low. |  |
| 6-2 |  | If a 16 x 16 link is connected to another device with an 8 x 8 HyperTransport link (the 16 x 16 link is only partially used), each of the unused CLK and CAD inputs of the link are pulled to a logic low, and the unused CTLIN[1] differential pair are pulled to logic high. For each pull up or pull down a 51- (± 5%) resistor is used. Note: For differential logic high, pull the true signal high and the complement low. For differential logic low, pull the true signal low and the complement high. |  |